# Project Name: CliniScan: Lung-Abnormality Detection on Chest X-rays

Submitted by: Ala Agastien

1. INTRODUCTION

CliniScan is an automated diagnostic system designed to identify 14 types of thoracic abnormalities from Chest X-rays. This report details the successful completion of Milestone 1, which focused on data preparation and initial setup. We have established a complete end-to-end data pipeline: transitioning from raw medical DICOM data to a fully validated training environment. The primary objective was to ensure high-fidelity image quality while conforming to the technical constraints of the YOLO architecture.

2. DATA PREPROCESSING (KAGGLE STAGE)

The initial stage involved large-scale data transformation to resolve the incompatibility between medical imaging formats and deep learning frameworks.

2.1 Image Standardization (DICOM → JPEG)

Medical DICOM files contain high-bit depth data and complex metadata. These were standardized for model ingestion:

- Pixel Extraction: Reading the 12-bit pixel array from DICOM using pydicom.

- VOI LUT Application: Applying Value of Interest Look-Up Tables to ensure medical visibility and contrast.

- Normalization: Scaling pixel intensities to the standard [0, 255] range.

- Resizing: Scaling to 1024 x 1024 pixels to balance diagnostic detail with computational speed.

- Storage Optimization: Files were saved as JPG (95% quality), reducing the total dataset size from 200GB+ to approximately 5.8 GB.

2.2 Annotation Transformation (CSV → YOLO)

The original annotations were provided in a central CSV file with absolute pixel coordinates. These were converted to the YOLO format:

- Result: A unique .txt file was generated for every image containing the class index and normalized coordinates.

3. ENVIRONMENT & DATASET INTEGRATION (COLAB STAGE)

After conversion, the project shifted to Google Colab to leverage high-performance GPU runtimes, specifically the NVIDIA Tesla T4.

3.1 Environment Configuration

- Storage Integration: Google Drive was mounted to persist the processed dataset.

- Performance Optimization: ZIP files were copied and extracted directly to the Colab local runtime to prevent I/O bottlenecks, reducing epoch time by approximately 60%.

- Library Stack: The environment was provisioned with ultralytics (YOLO), PyTorch, OpenCV, and Pandas.

4. VALIDATION & CONFIGURATION

4.1 Quality Assurance (QA)

- Parity Check: Confirmed a 1:1 ratio between image files and label files.

- Visual Audit: Random samples were rendered with bounding boxes overlaid to ensure the transformation from CSV to YOLO was 100% accurate.

- Negative Sample Handling: For images labeled "No finding," empty .txt files were created. This teaches the model to recognize healthy lungs and reduces the False Positive Rate.

4.2 YOLO Configuration (data.yaml)

- A data.yaml file was created to define the class mapping for the 14 abnormalities and the directory paths for the training and validation sets.

5. CHALLENGES & SOLUTIONS

- Storage Constraints: Resizing to 1024px and using JPG allowed the dataset to fit within the limits of Kaggle and Colab.

- Training Speed: Optimized data throughput by moving files from cloud storage to local SSD runtimes before training.

- Incompatibility: Successfully bridged the gap between medical DICOM standards and standard Computer Vision formats.

6. CONCLUSION

- Milestone 1 has successfully bridged the gap between medical raw data and deep learning requirements. The CliniScan environment is now ready for the modeling phase, focusing on baseline training and detection.